| Command | NONE | 1 | 2 | Documented Range | Comments |
| --- | --- | --- | --- | --- | --- |
| ADV | X |  |  | Advance to next scan freq. from Dwl | NOT when Scan Stopped |
| AGC(?) |  | X |  | 0 – 2 | Manual, Slow, Fast |
| BFO(?) |  | X |  | -8000 to +8000 | KHz in 10 Hz steps |
| BLK(?) |  | X |  | 00 – 10 mS | Noise Blanking Time |
| BWS(?) |  | X |  | 1 – 5 | 0.3, 1.0, 3.2, 6.0, 16.0 kHz |
| CHA(?) |  | X |  | 00 – 98 | Start Channel for Scan |
| CHB(?) |  | X |  | 01 – 99 | Stop Channel for Scan |
| CHI |  | X |  | 00 – 99 | Include Channel in Scan |
| CHS |  | X |  | 00 – 99 | Skip Channel in Scan |
| CLM | X |  |  |  | Clear all memories. |
| CTL(?) |  | X |  | 0 – 2, 0 not valid | Control Mode |
| DET(?) |  | X |  | 1 – 6 | AM, FM, CW, USB, LSB, ISB |
| ENA | X |  |  |  | Continue suspended scan |
| EXE |  | X |  | 0 – 99 | Recall and set Rx to mem. Channel |
| FRA(?) |  | X |  | 00.000000 - 29.999999 MHz | Frequency Scan Start |
| FRB(?) |  | X |  | 00.000001 - 30.000000 MHz | Frequency Scan Stop |
| FRQ(?) |  | X |  | 00.000000 - 30.000000 MHz | Set Rx Frequency |
| INC(?) |  | X |  | 0.001 - 25.000 (kHz) | Scan Inc. [0.0001 gives 01.000] |
| LCK |  |  | X | 00 – 99, Frequency | Set Lockout Frequency Channel |
| MUT? | X |  |  | 0 = Unmuted, 1 = Muted | Audio Mute |
| OPR(?) |  | X |  | 0 = Manual, 1 = Scan | Operating Mode |
| PBT(?) |  | X |  | -2000 to +2000 kHz | Pass Band Tuning Offset (in 10 Hz) |
| RCL? / STO |  | X |  | 00 – 99 (10 Params Return by RCL?) | Store/Recall Mem. Channel |
| REF? | X |  |  | 0 – 4 (0 = Int. 1 – 3 = Ext.) | Int./Ext. Frequency Reference |
| RFG(?) |  | X |  | 000 – 127 (0 - 100dB) | RF Gain |
| RFP(?) |  | X |  | 1 – 3 (Normal, Atten., Preamp) | No Preamp if FRQ < 500 kHz |
| RLK? |  | X |  | 00 - 99 | Recall Lockout Channel |
| SCF(?) |  | X |  | 1 - 3 | Scan Type(CH, F1-F2, w/lockout) |
| SCS? | X |  |  | 0=Manual,1=Scan,2=Dwell,3=Pause | Scan Status |
| SDW(?) |  | X |  | 0.5 – 20 Seconds (in 0.1 Sec.Incre.) | Scan Dwell Time (00.0 is Infinite) |
| SGV? | X |  |  | +20 to –135 dBm, 0 - 1 | Signal Level & Squelch(0 = On) |
| SLM? | X |  |  | 000 - 100 | Unused Lockout Channels |
| SPK(?) |  | X |  | 1 = USB, 2 = Both, 3 = LSB | Sound Selection |
| SQL(?) |  | X |  | 000 – 136 (-dBm) | Squelch Level, 136 = Off |
| SUS | X |  |  |  | Suspend scan. |
| ULK |  | X |  | 00 – 99 | Unlock Lockout Channel |
| CDE? | X |  |  | 00000 - 65535 | 5 digits returned |
| LDE? | X |  |  | 00000 – 65535 | 5 digits returned |
| *CLS | X |  |  |  | Clear all coms status registers |
| *ESE(?) | X |  |  | 00000 – 00255 | 5 digits returned |
| *ESR? |  | X |  | 00000 – 00255 | 5 digits returned |
| *IDN? | X |  |  | Equipment dependant | Returns 3 Parameters |
| *LRN? | X |  |  |  | Returns 9 Parameters |
| *OPC(?) | X |  |  | Request Op. Com. Return | 1 = Operation Complete |
| *RSE(?) |  | X |  | Rx status enable. | 5 digits returned |
| *RSR? | X |  |  | 00000 – 00255 | 5 digits returned |
| *RST | X |  |  |  | Reset |
| *SRE(?) |  | X |  | 00000 – 00255 | 5 digits returned |
| *STB? | X |  |  | 00000 – 00255 | 5 digits returned |
| *TST? | X |  |  | 00000 – 65535 | 00000 Returned if OK |

## General Receiver Control

| # Transmit | # Parameters |  | # Handbook |  |
| --- | --- | --- | --- | --- |
| # Command | # Tx | # Rx | # Documented Range | Comments |
| AGC(?) | 1 | 1 | 0 - 2 | Manual, Slow, Fast |
| BFO(?) | 1 | 1 | -8000 to +8000 | kHz in 10 Hz steps |
| BLK(?) | 1 | 1 | 00 – 10 (mS) | Noise Blanking Time |
| BWS(?) | 1 | 1 | 1 - 5 | 0.3, 1.0, 3.2, 6.0, 16.0 kHz |
| DET(?) | 1 | 1 | 1 - 6 | AM, FM, CW, USB, LSB, ISB |
| EXE | 1 | N/A | 00 – 99 | Recall & Set Rx to Memory Channel |
| FRQ(?) | 1 | 1 | 00.000000 – 30.000000 (MHz) | Set Rx Frequency |
| OPR(?) | 1 | 1 | 0 = Manual, 1 = Scan | Operating Mode |
| PBT(?) | 1 | 1 | -2000 to +2000 (Hz) | Pass Band Tuning Offset in 10 Hz |
| RFG(?) | 1 | 1 | 000 – 127 (0 – 100dB) | RF Gain |
| RFP(?) | 1 | 1 | 1 – 3 (Normal, Atten., Preamp.) | No Preamp if FRQ < 500 kHz |
| SGV? | N/A | 2 | +020 to –135 (dBm), 0 – 1 | Rx Signal Level & Squelch (0 = On) |
| SPK(?) | 1 | 1 | 1 = USB, 2 = Both, 3 = LSB | Speaker & DC Audio Selection |
| SQL(?) | 1 | 1 | 000 – 136 (-dBm) | Squelch Level (136 = Off) |
| *LRN? | N/A | 9 |  | Current Rx Settings |
| *OPC(?) | 1 | 1 | Request Op. Com. Return | OPC 1 = Operation Complete |

## Channel Control

| # Transmit |  | # Parameters |  | # Handbook |  |
| --- | --- | --- | --- | --- | --- |
| # Command |  | # Tx | # Rx | # Documented Range | Comments |
| ADV | N/A |  | N/A |  | Advance to next Scan from Dwell |
| CHA(?) | 1 |  | 1 | 00 – 98 | Start Channel for Scan |
| CHB(?) | 1 |  | 1 | 01 – 99 | Stop Channel for Scan |
| CHI | 1 |  | N/A | 00 – 99 | Include Channel in Scan |
| CHS | 1 |  | N/A | 00 – 99 | Skip Channel in Scan |
| CLM | N/A |  | N/A |  | Clear All Memories |
| ENA | N/A |  | N/A |  | Continue Suspended Scan |
| OPR(?) | 1 |  | 1 | 0 = Manual, 1 = Scan | Operating Mode |
| RCL ? | 1 |  | 10 | 00 – 99 | Recall Memory Channel Settings |
| SDW(?) | 1 |  | 1 | 0.5 – 20.0 (Seconds in 0.1 Sec. Inc.’s) | Scan Dwell Time (00.0 = Infinite) |
| STO | 1 |  | N/A | 00 – 99 | Store Rx Settings to Memory Channel |
| SCF( ?) | 1 |  | 1 | 1 = Chan. 2 = F1-F2 3 = F1-F1w/Lckout | Scan Type |
| SCS ? | N/A |  | 1 | 0 = Manual, 1 = Scan, 2 = Dwell, | 3 = Pause (Scan Status) |
| SUS | N/A |  | N/A |  | Suspend Scan |
| *OPC(?) | N/A |  | 1 | Request Op. Com. Return | OPC 1 = Operation Complete |

RCL? nn   RCL  nn,0,20.000000,2,1,4,136,1,+1000,000   for a reset channel

All ? type commands do not return the ?.

### Frequency Scan

| # Transmit |  | # Parameters |  | # Handbook |  |
| --- | --- | --- | --- | --- | --- |
| # Command |  | # Tx | # Rx | # Documented Range | Comments |
| ADV |  | N/A | N/A |  | Advance to next Scan from Dwell |
| CLM |  | N/A | N/A |  | Clear All Memories |
| ENA |  | N/A | N/A |  | Continue Suspended Scan |
| FRA(?) |  | 1 | 1 | 00.000000 – 29.999999 (MHz) | Frequency Scan Start |
| FRB(?) |  | 1 | 1 | 00.000001 – 30.000000 (MHz) | Frequency Scan Stop |
| INC(?) |  | 1 | 1 | 00.001 – 25.000 (kHz) | Frequency Scan Increment |
| LCK |  | 2 | N/A | 00 – 99, Frequency | Set Lockout Frequency Channel |
| OPR(?) |  | 1 | 1 | 0 = Manual, 1 = Scan | Operating Mode |
| RLK? |  | 1 | 2 | 00 – 99,Frequency | Recall Frequency Lockout Channel |
| SCF(?) |  | 1 | 1 | 1 = Chan. 2 = F1-F2 3 = F1-F1w/Lckout | Scan Type |
| SCS? |  | N/A | 1 | 0 = Manual, 1 = Scan, 2 = Dwell, | 3 = Pause (Scan Status) |
| SDW(?) | 1 |  | 1 | 0.5 – 20.0 (Seconds in 0.1 Sec. Inc.’s) | Scan Dwell Time (00.0 = Infinite) |
| SLM? |  | N/A | 1 | 000 – 100 | Unused Frequency Lockout Channels |
| SUS |  | N/A | N/A |  | Suspend Scan |
| ULK |  | 1 | N/A | 00 – 99 | Unlock Frequency Lockout Channel |
| *OPC(?) |  | N/A | 1 | Request Op. Com. Return | OPC 1 = Operation Complete |

## Receiver Registers

| # Transmit | # Parameters |  | # Handbook |  |
| --- | --- | --- | --- | --- |
| # Command | # Tx | # Rx | # Documented Range | Comments |
| CDE? | N/A | 1 | 00000 – 65535 | 5 digits returned |
| LDE? | N/A | 1 | 00000 – 65535 |  |
| *CLS | N/A | N/A |  | Clear all coms status registers |
| *ESE(?) | 1 | 1 | 00000 – 00255 | 5 digits returned |
| *ESR? | N/A | 1 | 00000 – 00255 | 5 digits returned |
| *RSE(?) | 1 | 1 | Rx status enable. | 5 digits returned |
| *RSR? | N/A | 1 | 00000 – 00255 | 5 digits returned |
| *SRE(?) | 1 | 1 | 00000 – 00255 |  |
| *STB? | N/A | 1 | 00000 – 00255 |  |

Misc. Commands

| # Transmit | # Parameters |  | # Handbook |  |
| --- | --- | --- | --- | --- |
| # Command | # Tx | # Rx | # Documented Range | Comments |
| CTL(?) | 1 | 1 | 0 – 2 (0 not valid) | Remote Control Mode |
| MUT? | N/A | 1 | 0 = Unmuted, 1 = Muted | External Rx Mute |
| REF? | N/A | 1 | 0 – 4 (0 = Int. 1 – 3 = Ext.) | Int./Ext. Frequency Reference |
| *IDN? | N/A | 3 | Equipment ID | Returns Equip. Info |
| *RST | N/A | N/A |  | Reset Rx |
| *TST? | N/A | 1 | 0 – 65535 | 00000 Returned if OK |

*IDN?     *IDN WJ8712,0,03.00.01